MCS Lab 1 Report

Sakka Mohamad-Mario

el-ghoul Layla

mahmoud mirghani abdelrahman

MCS

# 1. Plots resulting from code

Figure 1: Plot of free space propagation loss as a function of distance with frequency as a parameter

Figure 2: Plot of free space propagation loss as a function of distance with frequency as a parameter, using a logarithmic scale for distance.

Figure 3: Plot of free space propagation loss as a function of frequency with distance as a parameter, using a logarithmic scale for frequency.

# 2. Answering questions

Q1. Determine the slope for free space propagation losses variation as a function of frequency.

We know the FSPL expression in dB has a 20log⁡10f term, so the loss grows at 20 dB/decade (≈6.02 dB per doubling). Since FrSpcLss returns link gain (which is negative and gets more negative as fincreases), the change in gain per doubling is about −6.02 dB, confirming the 20 dB/decade magnitude.

The following MATLAB code plots the FrSpcLss function with constant distance and antenna gains, so it remains a function of frequency, and it allows us to visualize the slope of 20dB/decade. It also computes the difference in link gain between two frequencies, one of them being double the other.

function q1_slope_freq

% Q1: Slope vs frequency using FrSpcLss (returns link gain in dB).

c  = 3e8;              %#ok<NASGU> % not used directly, but kept for completeness

d  = 1000;             % 1 km

Gt = 1; Gr = 1;        % isotropic (linear gains)

f  = logspace(6,9,400);                  % 1 MHz ... 1 GHz

Lg = arrayfun(@(ff) FrSpcLss(d, ff, Gt, Gr), f);  % link gain [dB]

% Slope check (doubling frequency)

f1 = 100e6; f2 = 2*f1;

L1 = FrSpcLss(d, f1, Gt, Gr);

L2 = FrSpcLss(d, f2, Gt, Gr);

fprintf('Q1: Δ(link gain) for doubling f = %.2f dB (expected ≈ -6.02 dB)\n', L2-L1);

figure; semilogx(f, Lg); grid on;

xlabel('Frequency [Hz]'); ylabel('Link gain (FrSpcLss) [dB]');

title('Q1: Link gain vs frequency at d = 1 km (|slope| ≈ 20 dB/dec)');

hold on; plot([f1 f2],[L1 L2],'x-');

text(f1,L1,sprintf('  %.1f dB @ 100 MHz',L1));

text(f2,L2,sprintf('  %.1f dB @ 200 MHz',L2));

end

Q2. Determine the slope for free space propagation losses variation as a function of distance.

FSPL also has a 20log⁡10d term, so loss increases 20 dB/decade with distance (≈6.02 dB per doubling). Because FrSpcLss outputs gain, we expect the gain to drop by about 6.02 dB each time distance doubles. Here’s the code that confirms it:

function q2_slope_dist

% Q2: Slope vs distance using FrSpcLss (returns link gain in dB).

f  = 900e6;           % 900 MHz

Gt = 1; Gr = 1;

d  = logspace(0,4,400);               % 1 m ... 10 km

Lg = arrayfun(@(dd) FrSpcLss(dd, f, Gt, Gr), d);

% Slope check (doubling distance)

d1 = 100; d2 = 2*d1;

L1 = FrSpcLss(d1, f, Gt, Gr);

L2 = FrSpcLss(d2, f, Gt, Gr);

fprintf('Q2: Δ(link gain) for doubling d = %.2f dB (expected ≈ -6.02 dB)\n', L2-L1);

figure; semilogx(d, Lg); grid on;

xlabel('Distance [m]'); ylabel('Link gain (FrSpcLss) [dB]');

title('Q2: Link gain vs distance at f = 900 MHz (|slope| ≈ 20 dB/dec)');

hold on; plot([d1 d2],[L1 L2],'x-');

text(d1,L1,sprintf('  %.1f dB @ 100 m',L1));

text(d2,L2,sprintf('  %.1f dB @ 200 m',L2));

end

Q3. What can be observed in the proximity of the transmitter (distances ≈ wavelength)?

Friis/FSPL assume far-field operation, i.e., distances much larger than λ. Around a few wavelengths, near-field effects mean the simple 1/d2 behavior isn’t reliable, near-field effects appear, reactive fields dominate, Friis equation is not accurate. We mark λ and 10λ at 900 MHz to show that boundary.

function q3_nearfield_demo

% Q3: Visual near-field indication with colored markers at λ and 10λ.

c  = 3e8;

f  = 900e6;

lambda = c/f;

Gt = 1; Gr = 1;

d  = logspace(-2,3,500);               % 1 cm ... 1 km

Lg = arrayfun(@(dd) FrSpcLss(dd, f, Gt, Gr), d);

fprintf('Q3: At 900 MHz, λ = %.3f m. Friis/FSPL not reliable for d ~ few·λ.\n', lambda);

figure; semilogx(d, Lg); grid on;

xlabel('Distance [m]'); ylabel('Link gain (FrSpcLss) [dB]');

title('Q3: Link gain vs distance with markers at \lambda and 10\lambda (900 MHz)');

% Colored markers

xline(lambda,   '-', '  \lambda',  'Color', [1 0 0], 'LineWidth', 1.5);   % red

xline(10*lambda,'-', '  10\lambda','Color', [0 0 1], 'LineWidth', 1.5);   % blue

end

Q4. Determine the propagation losses in the case when the distance between the transmitter and the receiver is d =1 km, the frequency of the signal is f = 900 MHz and the antennae have a gain of 2 dB. Indication: use the Matlab function FrSpcLss.

FrSpcLss gives us link gain in dB. To report loss, we take the negative of that result. We include the antenna gains by converting 2 dB to linear, calling FrSpcLss, and then computing Lloss = - Lg

function q4_loss_numeric

% Q4: Compute path loss at 1 km with GT=GR=2 dB, using FrSpcLss gain -> loss=-gain.

f    = 900e6;

d    = 1000;       % 1 km

GTdB = 2; GRdB = 2;

Gt   = 10^(GTdB/10);

Gr   = 10^(GRdB/10);

Lg_1km = FrSpcLss(d, f, Gt, Gr);   % link gain [dB]

L_loss = -Lg_1km;                  % path loss [dB]

fprintf('Q4: Path loss at 1 km, 900 MHz, GT=GR=2 dB: %.2f dB\n', L_loss);

% Plot loss vs distance (1 m .. 10 km) and mark 1 km

dvec = logspace(0,4,400);

Lvec = arrayfun(@(dd) -FrSpcLss(dd, f, Gt, Gr), dvec);  % compute loss inline

figure; semilogx(dvec, Lvec); grid on;

xlabel('Distance [m]'); ylabel('Path loss [dB]');

title('Q4: Path loss vs distance at 900 MHz (GT=GR=2 dB)');

hold on; plot(d, L_loss, 'x'); text(d, L_loss, sprintf('  %.1f dB @ 1 km', L_loss));

end

Q5. Consider a radio transmitter with PT= 50 W. If the antenna of the transmitter is isotropic and the frequency of the emitted signal is 900 MHz, compute the free space received power at the distance d=100m from the transmitter. It is supposed that the receiver also has an isotropic antenna.

FrSPcLss returns 10log⁡10(PR/PT). So, we recover PRvia PR=PT⋅10FrSpcLss(d,f,GT,GR)/10. With PT=50W, d=100m, f=900MHz, isotropic gains, we expect around 3.5.

function q5_received_power_demo

% Q5: Received power using FrSpcLss directly.

PT  = 50;           % W

f   = 900e6;        % Hz

Gt  = 1; Gr = 1;    % isotropic

dMk = 100;          % m

LgMk   = FrSpcLss(dMk, f, Gt, Gr);      % dB of (PR/PT)

PR_Mk  = PT * 10^(LgMk/10);             % W

PR_dBm = 10*log10(PR_Mk/1e-3);

fprintf('Q5: PR @100 m, 900 MHz, PT=50 W (iso): %.3e W = %.2f µW (%.2f dBm)\n', ...

PR_Mk, PR_Mk*1e6, PR_dBm);

% Curve PR vs distance (10 m .. 10 km)

d   = logspace(1,4,400);

Lg  = arrayfun(@(dd) FrSpcLss(dd, f, Gt, Gr), d);

PR  = PT .* 10.^(Lg/10);

figure; loglog(d, PR); grid on;

xlabel('Distance [m]'); ylabel('Received Power [W]');

title('Q5: Received power vs distance (PT=50 W, f=900 MHz, isotropic)');

hold on; plot(dMk, PR_Mk, 'x');

text(dMk, PR_Mk, sprintf('  %.2f µW @ 100 m', PR_Mk*1e6));

end

Q6. If the power received by an antenna having the gain  is , and the frequency of the signal is f=900MHz, determine the intensity of the electrical field at the receiver. Indication: relation (3) should be used.

We used the lab relations Ae=GRλ24π, S=PRAe, and S=E2η⇒E=Sη with η≈377. Plugging PR, GR, and fgives Ein V/m.

function q6_field_from_power_demo

% Q6: Compute |E| from PR, GR, f (direct formulas; no helpers).

c   = 3e8;          % [m/s]

eta = 377;          % [ohm]

PR  = 7e-10;        % [W]

GR  = 2;            % linear power gain

f   = 900e6;        % [Hz]

lambda = c / f;

Ae = GR * lambda^2 / (4*pi);   % effective aperture [m^2]

S  = PR / Ae;                   % power flux density [W/m^2]

E  = sqrt(S * eta);             % field magnitude [V/m]

fprintf('Q6: E ≈ %.4f V/m = %.2f mV/m (at 900 MHz, GR=2, PR=7e-10 W)\n', E, E*1e3);

% Visual: E vs PR with the given point

PR_range = logspace(-12,-6,400);

Ae_const = Ae;                  % constant for fixed GR and f

E_curve  = sqrt((PR_range./Ae_const) * eta);

figure; semilogx(PR_range, E_curve); grid on;

xlabel('Received Power PR [W]'); ylabel('E-field magnitude [V/m]');

title('Q6: E-field vs PR (GR=2, f=900 MHz)');

hold on; plot(PR, E, 'x'); text(PR, E, sprintf('  %.2f mV/m @ %.1e W', E*1e3, PR));

end